HPPRINT is a small program to send data to a HP printer connected via

network and a HP JetDirect card (in the printer). It can be used

together with PRINTMON to redirect an OS/2 printer queue to the

network connected printer. A sample usage is:

PRINTMON lpt1 "HPPRINT -h laser"

where lpt1 is the port to redirect and laser is the "host name" of

your printer. It must be defined in your hosts file or by your domain

name server. Alternatively it can be an IP address in dotted-decimal

notation. Optionally, the program accepts a port number with the -p

option, it can be either a port name defined in the services file or a

directly specified number.

Note that to get your printer working you must have a bootpd server

daemon running on some machine in the LAN and have a proper definition

for your printer in the bootptab file. Example:

global.dummy:\

:sm=255.255.255.0:\

:hd=/bootpd/trypd:bf=null:\

:ds=1.1.1.6:

laser.your.domain: tc=global.dummy: ht=ethernet:\

ha=080009568a6f: ip=1.2.3.4: hn:

Alternatively, the HP JetDirect interface can be configured to

a fixed IP address using the printer's control panel.

Please don't ask me in case of problems, there can be too many. Make

sure you can ping your printer before attempting to use HPPRINT.

The program requires the EMX runtime DLL's, get the latest emxrt.zip

which contains it. It also requires the IBM TCP/IP 2.0 socket DLL's

(tcp32dll.dll and so32dll.dll).

This program is freeware. The author is not responsible in any way for

any problem caused by it, even if the program is used correctly.

Kai Uwe Rommel

--

/* Kai Uwe Rommel   ARS Computer & Consulting GmbH, Muenchen, Germany *

* rommel@ars.de          CompuServe 100265,2651, Fax +49 89 324 4524 *

* rommel@leo.org (ftp://ftp.leo.org/pub/comp/os/os2 maintenance)     */

DOS ... is still a real mode only non-reentrant interrupt

handler, and always will be.                -Russell Williams